REFERENCE SYSTEM  ·  MULTI-MISSION PLATFORM

# AR-7

Long-Endurance Hybrid-Frame Quadcopter  ·  320mm 7-inch class

# Executive Summary

The AR-7 is a 7-inch class long-endurance quadcopter optimized for ISR, communications relay, mapping, and repeated logistics missions. The platform employs a hybrid-frame architecture — 3D-printed body components combined with pultruded carbon fiber tube arms — to deliver production-grade reliability while remaining buildable on consumer-grade equipment at a kitchen-table workstation.

With 25–35 minutes of typical mission endurance, an honest 2–3 lb payload capacity, and a per-unit BOM of $280–360 excluding battery and payload, the AR-7 fills the workhorse role between the Disposable Aedes MD's single-use class and the HX-1 Heavy Hex.

# Mission Profile & Use Cases

## Primary Missions

- Sustained ISR over fixed or mobile targets

- Communications relay — mesh networking, UHF/VHF repeater extension

- Multispectral mapping and terrain modeling for disaster response and infrastructure assessment

- Repeated logistics delivery of medical supplies, equipment, or critical components

- Infrastructure inspection for utilities, agriculture, search and rescue, and emergency response

## Operational Envelope

| Endurance (no payload) | 35–45 min cruise |
| --- | --- |
| Endurance (2 lb payload) | 18–25 min |
| Top speed (clean) | 70–80 mph |
| Top speed (with payload) | 50–60 mph |
| Payload capacity | 2–3 lb sustained, honest |
| Operating range | 5–10 km (radio-link dependent) |

# Design Philosophy: Hybrid Frame Architecture

Three forces destroy a fully-3D-printed airframe at the 7-inch class:

- Vibration fatigue — brushless motor harmonics cause progressive micro-fracturing along print layer lines

- Heat soak — sustained ESC and motor temperatures exceed PETG glass transition (~75°C) on summer operations

- Crash recovery — a bent monocoque is a full reprint and rebuild; a bent CF tube is a $5 swap completed in 10 minutes

The hybrid architecture resolves all three failure modes:

- 3D-printed body parts (top plate, bottom plate, motor mount housings, GPS standoff) handle low-stress structural roles where print methods perform well

- Pultruded carbon fiber square tubes (16×16mm × 110mm) carry the high-stress arm load — vibrationally inert, thermally stable, mechanically superior

- M3 fasteners throughout — no snap-fits, no friction joints in any load path

## Production Parallelism

The hybrid architecture is more distributable than monocoque printing — the workflow parallelizes naturally and matches the Aedes builder network model precisely:

- Builder A — body part prints (4-part build, parallelizable across multiple printers)

- Builder B — tube prep (cut, chamfer, deburr 4 arms in under 10 minutes per drone)

- Builder C — electronics sub-assembly (motors-to-ESCs soldering, FC integration, pre-flashed firmware)

- Final integration station — mate components, configure firmware, bench test and tune

# Full Specifications

| DESIGNATION | AR-7 |
| --- | --- |
| Configuration | Quadcopter, 320mm X-frame |
| Mission profile | Multi-mission ISR / logistics / mapping |
| Frame architecture | Hybrid: 3D-printed body + CF tube arms |
| Body material | PETG-CF or ABS-X (vanilla PETG acceptable) |
| Body print time | 6–8 hours total across 4 parts |
| Arms | 4× 16mm pultruded CF square tube, 110mm length |
| Motors | 2806.5 brushless, 1500–1800kv (6S) |
| Propellers | 7-inch tri-blade (HQProp 7×4×3 class) |
| Battery | 6S 8000mAh LiPo (Li-ion alternative supported) |
| All-up weight (dry) | ~750–850 g |
| All-up weight (with battery) | ~1500 g |
| All-up weight (with 2 lb payload) | ~2400 g |
| Top speed (clean) | 70–80 mph |
| Top speed (with 2 lb payload) | 50–60 mph |
| Endurance (no payload) | 35–45 min cruise |
| Endurance (2 lb payload) | 18–25 min |
| Operating range | 5–10 km (radio-link dependent) |
| Payload interface | Universal quick-release rail, bottom-mount |
| Flight controller | F7-class — ArduPilot or Betaflight |
| Radio link | ELRS 915 MHz |
| Video | HDZero or Walksnail digital (US-friendly) |
| GPS / compass | M10-class with mast standoff |
| First build time | 12–15 hours |
| Experienced build time | 4–6 hours |
| Total BOM | $280–360 (excluding battery and payload) |
| Design status | Reference specification — under production validation |

# Bill of Materials

Per-unit BOM for a complete AR-7 platform, excluding battery and payload module. All components sourced from US-domestic suppliers where available.

| COMPONENT | SPECIFICATION | EST. COST |
| --- | --- | --- |
| Frame body | PETG-CF filament, ~250g across 4 parts | $15 |
| CF tube arms (×4) | 16×16mm pultruded, 110mm cut length | $20 |
| Hardware kit | M3 bolts, nylock nuts, standoffs, vibration pads | $8 |
| Motors (×4) | 2806.5 1500–1800kv brushless | $120 |
| Propellers (×8) | 7-inch tri-blade (4 + 4 spares) | $16 |
| 4-in-1 ESC | 60A BLHeli32 / AM32, 6S-rated | $45 |
| Flight controller | F7-class, ArduPilot/Betaflight compatible | $50 |
| GPS / compass | M10-class with mast | $25 |
| Radio receiver | ELRS 915 MHz | $15 |
| Video transmitter | HDZero or Walksnail (digital) | $60 |
| FPV camera | Digital camera (paired to VTX) | $30 |
| Wiring / connectors | XT60, silicone wire, capacitors, heat shrink | $10 |
| TOTAL PER UNIT | (excluding battery and payload) | ~$414 |

Note: BOM total reflects retail single-unit pricing. At network scale (volume sourcing of motors, ESCs, FCs), the per-unit cost falls into the $280–360 range cited in the executive summary.

# Build Process & Production Parallelization

Total first-build time is 12–15 hours; experienced builders in the network reach 4–6 hours per unit. The build is structured to parallelize across a multi-builder cell:

## Stage 1 — Print & Tube Prep (Parallel)

- Print 4 body components in PETG-CF: top plate, bottom plate, motor mount housings, GPS mast

- Cut 4 CF tubes to 110mm length using a $15 pipe cutter

- Chamfer and deburr tube ends; bond optional vibration-isolation sleeves

## Stage 2 — Electronics Sub-Assembly

- Solder motor leads to 4-in-1 ESC pads

- Stack flight controller on ESC, connect signal and power wiring

- Pre-flash ArduPilot or Betaflight firmware with AR-7 reference profile

- Test bench-level functionality before integration

## Stage 3 — Frame Integration

- Insert CF tubes into printed motor mount housings, secure with M3 cross-bolts

- Mount motors to housings using M3 bolts (4 per motor)

- Sandwich electronics stack between top and bottom plates with M3 standoffs

- Install GPS mast, video transmitter, antennas, payload rail

## Stage 4 — Configuration & Test

- Calibrate accelerometer, gyro, compass, and ESCs

- Bind radio receiver to ground transmitter

- Bench test: motor direction check, throttle response, failsafe behavior

- First-flight check: hover trim, control surface response, telemetry confirmation

# Performance Envelope

AR-7 performance is published as a tradeoff envelope rather than a list of peak numbers. Speed, payload, and endurance interact — choose two and the third is constrained.

| PROFILE | RESULT |
| --- | --- |
| Clean cruise (no payload) | 70–80 mph top speed, 35–45 min endurance |
| Loaded cruise (2 lb payload) | 50–60 mph top speed, 18–25 min endurance |
| Maximum payload (3 lb) | Reduced agility, 12–18 min endurance, 40–50 mph top |
| Long-range loiter (no payload) | 30 mph cruise, ~50 min endurance, ~25 km radius |
| ISR station-keep (1 lb sensor) | Hover/loiter, 22–28 min on-station |

# Cost Analysis

## Unit Economics

| Retail BOM (single unit) | ~$414 |
| --- | --- |
| Volume BOM (network sourcing) | $280–360 |
| Build labor (network rate) | 4–6 hours experienced |
| All-in production cost (at scale) | $450–600 per unit |
| Per-flight cost (amortized over 200-flight life) | $2.25–3.00 + battery wear |

## Mission Cost Comparisons

| Commercial inspection drone (DJI Matrice class) | $15,000–25,000 unit cost |
| --- | --- |
| Per-flight cost — DJI Matrice | $50–100 amortized |
| AR-7 unit cost | $450–600 |
| AR-7 per-flight cost | $2.25–3.00 amortized |

# Compliance & Limitations

## Regulatory

- Subject to FAA Part 107 commercial drone rules in the United States

- BVLOS operations require Part 107 waiver or Part 108 authorization

- Remote ID compliant when configured with appropriate broadcast module

- International deployment subject to ITAR, EAR, and host-nation drone regulations

## Non-Weaponization

- Platform is designed for ISR, communications, mapping, and logistics applications

- Use for weapons delivery, weapons integration, or prohibited applications is not authorized under the Aedes Network Builder Agreement

- Payload interface is restricted to authorized cargo and sensor types per Aedes Compliance Policy

## Operational Limits

- Designed for VFR conditions; precipitation reduces range and endurance significantly

- Operating temperature: -10°C to 45°C ambient

- Maximum density altitude: 12,000 ft MSL

- Wind tolerance: stable hover up to 25 mph sustained, 35 mph gusts

- Service life: 200+ flights with routine maintenance (motor bearings, propeller replacement)

# Design Status

Reference specification, version 1.0. Subject to refinement through production validation pilots. Final specifications will be confirmed following first-article inspection and flight test of network-built units.