REFERENCE SYSTEM  ·  DISPOSABLE VARIANT

# Disposable Aedes MD

Single-Use Autonomous Aerial Platform  ·  250mm 5-inch class

# Executive Summary

The Disposable Aedes MD is a single-mission autonomous UAV optimized for one-way logistics, sensor deployment, and disaster response missions where airframe recovery is uneconomical or unsafe. Built on the 250mm 5-inch quadcopter class with a single-print PLA monocoque frame, the platform delivers 15–20 km of one-way range carrying up to 2 lb of payload at a fully-loaded unit cost of approximately $220.

The design treats non-recovery as a feature, not a compromise — releasing weight, complexity, and cost from every subsystem that exists only to support repeated flights. This produces a fundamentally different optimization curve from a multi-use platform, and a meaningfully lower cost-per-mission for the use cases it serves.

# Mission Profile & Use Cases

## Primary Missions

- One-way aid delivery into disaster zones, denied-access regions, or remote terrain (medical supplies, blood, vaccines, water purification)

- Sensor emplacement — atmospheric sampling in wildfires, hurricanes, or volcanic plumes; seismic sensor drops; environmental monitoring nodes

- Communications relay deployment — drone flies to high vantage point, lands intact, becomes static mesh node

- Search and rescue — radio beacon or supply drop into terrain inaccessible to ground teams

- Disposable ISR for hurricane damage assessment, wildfire perimeter mapping, post-disaster infrastructure recon

## Operational Envelope

| One-way cruise range | 15–20 km |
| --- | --- |
| Payload capacity | Up to 2 lb (900g) |
| Cruise speed | 35–45 mph |
| Endurance (cruise) | 25–30 min one-way |
| Launch-to-deployment | Under 60 seconds per unit |
| Volume operations | Single operator can launch 20+ units in sequence |

# Design Philosophy: Single-Use Optimization

Multi-use airframe design optimizes for durability, repairability, and fatigue life. Single-use design optimizes for unit cost and one-shot reliability. The Disposable Aedes MD inverts every multi-use priority where the inversion creates value:

- Durability past one flight is unnecessary, freeing material choices toward cost over longevity (PLA over PETG-CF, 15% infill over 30%)

- Crash-recovery is irrelevant — landing intact is a non-requirement except for the cargo

- Redundancy is removed — a single point of failure is acceptable at this unit cost

- FPV piloting is replaced with autonomous waypoint flight, eliminating video link hardware entirely

- PID tuning and trim are pre-loaded; field calibration is minimized to GPS lock and control surface check

## Non-Negotiable Engineering Standards

Some failure modes remain unacceptable regardless of mission lifetime. The following standards are retained from multi-use practice:

- M3 fasteners on motor mounts. A motor shedding mid-flight is a public-safety failure mode regardless of airframe lifetime. Snap-fit and friction-fit motor mounts are excluded.

- Positive-lock battery retention. A LiPo ejecting at cruise speed is a public-safety hazard.

- Inline thermal cutoff on the LiPo pack. Prevents post-impact battery fires, particularly important for wildfire-adjacent operations.

- Payload protection. The airframe is disposable; the cargo typically is not. Parachute deployment for cargo is standard equipment.

# Full Specifications

| DESIGNATION | DISPOSABLE AEDES MD |
| --- | --- |
| Configuration | Quadcopter, 250mm X-frame |
| Mission profile | Single-use, autonomous waypoint |
| Frame architecture | Single-print PLA monocoque, 15% infill |
| Frame material cost | Under $5 |
| Frame print time | 3–4 hours (consumer FDM, 200×200mm bed) |
| Motors | 2207 brushless, 1750kv (cruise-optimized) |
| Propellers | 5-inch tri-blade |
| Battery | 4S 2200mAh LiPo (or LiHV equivalent) |
| All-up weight (no payload) | ~520 g |
| All-up weight (max payload) | ~1.4 kg |
| Cruise speed | 35–45 mph |
| One-way range | 15–20 km |
| Endurance (cruise) | 25–30 min |
| Payload interface | Belly bay, servo-actuated release |
| Flight controller | F4-class, ArduPilot Auto mode |
| Radio link | ELRS 915 MHz (telemetry only, no FPV) |
| GPS / compass | M10-class with mast |
| Build time per unit | 3–4 hours |
| Total BOM (ex-payload) | $180–220 |
| Required tools | 3D printer, soldering iron, screwdriver set |
| Design status | Reference specification — under production validation |

# Bill of Materials

Per-unit BOM for a complete Disposable Aedes MD platform, excluding payload. All components sourced from US-domestic suppliers where available.

| COMPONENT | SPECIFICATION | EST. COST |
| --- | --- | --- |
| Frame | PLA filament, ~200g, 15% infill | $4 |
| Hardware kit | M3 bolts, nylock nuts, standoffs | $5 |
| Motors (×4) | 2207 1750kv brushless | $80 |
| Propellers (×8) | 5-inch tri-blade (4 + 4 spares) | $8 |
| 4-in-1 ESC | 35A BLHeli32 / AM32 | $25 |
| Flight controller | Speedybee F405 mini or equivalent | $30 |
| GPS / compass | M10-class with mast | $20 |
| Radio receiver | ELRS 915 MHz | $15 |
| Servo (payload release) | 9g micro servo | $4 |
| Battery | 4S 2200mAh LiPo | $25 |
| LiPo thermal fuse | Inline thermal cutoff | $2 |
| Wiring / connectors | XT60, silicone wire, heat shrink | $5 |
| TOTAL PER UNIT | (excluding payload) | ~$223 |

# Build Process

Total build time is 3–4 hours per unit for a network builder. The workflow is fully parallelizable — a 5-builder cell can sustain 8–10 units per day.

- Frame print (parallel). Single PLA print on a consumer FDM printer. 3–4 hours per frame, but builders run multiple printers simultaneously.

- Electronics sub-assembly. Solder motors to ESC, mount FC to ESC stack, integrate GPS and receiver. ~90 minutes per stack. This sub-assembly can be done in advance and stockpiled.

- Frame integration. Mount motors to frame using M3 bolts. Install electronics stack. Mount GPS mast. Install payload release servo.

- Flight controller configuration. Pre-loaded ArduPilot firmware with mission profile templates. Field operator sets waypoints; drone executes autonomously. No PID tuning required for cruise-only flight.

- Pre-launch test. Power-on self-test, GPS lock, control surface check, payload retention check. Less than 5 minutes per unit.

# Concept of Operations

## Mission Planning

- Operator defines GPS waypoint mission via ground control station (Mission Planner, QGroundControl)

- Pre-loaded mission profiles for common scenarios: delivery, sensor drop, comms relay deployment, ISR pass

- Multiple units can be queued for sequential or simultaneous launch from a single operator station

## Launch & Execution

- Hand or stand launch from any open area (5×5 m clearance sufficient)

- Auto-takeoff to programmed cruise altitude

- Autonomous waypoint navigation to drop point

- Payload release via servo-actuated belly bay at programmed coordinates

- Optional descent profile: parachute deployment for cargo, controlled glide to ground for airframe

- Telemetry confirmed via ELRS link to ground station

## Termination

- Airframe remains at drop site for retrieval (where possible) or accepts loss

- LiPo thermal cutoff prevents post-impact fire

- Mission complete on payload-release confirmation

# Cost Analysis

## Unit Economics

| Per-unit BOM | ~$220 |
| --- | --- |
| Per-unit build labor | 3–4 hours at network rate |
| All-in production cost (at scale) | $300–400 per unit |
| Cost per delivered mission | $300–400 + payload |

## Comparable Mission Costs

| Helicopter aid sortie | $5,000–20,000 |
| --- | --- |
| Zipline-class reusable network | $15–30 per delivery (requires fixed infrastructure) |
| Hand-carried delivery (dangerous areas) | Indeterminate — human risk premium |
| Disposable Aedes MD per-mission cost | $300–400 + payload |

## When the Disposable Aedes MD is Economically Rational

- Mission volume is low to moderate (insufficient to justify reusable network infrastructure)

- Operating environment is dangerous, denied, or remote (recovery is impractical)

- Time-to-deploy is critical (no fixed infrastructure setup required)

- Cargo value justifies the per-mission cost (medical supplies, sensor packages, comms hardware)

# Compliance & Limitations

## Regulatory

- Subject to FAA Part 107 commercial drone rules in the United States

- Beyond Visual Line of Sight (BVLOS) operations require Part 107 waiver or Part 108 authorization

- Autonomous operations are subject to airspace deconfliction requirements

- International deployment subject to ITAR, EAR, and host-nation drone regulations

## Non-Weaponization

- Platform is designed for non-weaponized applications consistent with the Aedes Network Builder Agreement

- Use for weapons delivery, weapons integration, or prohibited applications is not authorized

- Payload module is restricted to authorized cargo types per Aedes Compliance Policy

- Violations of the non-weaponization policy are referred to appropriate authorities

## Operational Limits

- Designed for VFR conditions; precipitation and high winds (>20 mph) significantly reduce range

- Operating temperature: 0°C to 40°C ambient (PLA frame thermal envelope)

- Maximum density altitude: 8,000 ft MSL

- Single-use platform — recovery and re-flight are not supported design states

# Design Status

Reference specification, version 1.0. Subject to refinement through production validation pilots. Final specifications will be confirmed following first-article inspection and flight test of network-built units.